Lab 2 – Traffic Light – Test Plan

- Turn the traffic light on, cycle through lights with no left turns

  - Start with NS Green for 3 counts

    - Count3 enabled

    - All other counters not enabled

    - Count3 term high sets up next state to “NS_Yellow”

  - Transition to  NS Yellow for 2 counts

    - When the state changes:

      - Count3_en = 0

      - Count3_term = 0

      - Count2_en = 1

    - Count2 term high sets up next state to “EW_Green”

  - Then EW Green for 3 counts

    - When state changes

      - Count3_en = 1

      - Count2_en = 0

      - Count2_term = 0

    - Count3 term high sets up next state to “EW_Yellow”

  - Then EW Yellow for 2 counts

    - When the state changes:

      - Count3_en = 0

      - Count3_term = 0

      - Count2_en = 1

    - Count2 term high sets up next state to “NS_Green”

A

B

C

D

E

Then go back to NS Green

- Cycle through lights with NS Left Turn bit on

  - Start with NS Green

    - Count3 enabled

    - Count3 term high sets up next state to “NS_Yellow”

  - Transition to  NS Yellow for 2 counts

  - Then EW Green for 3 counts

  - Then EW Yellow for 2 counts

    - When the state changes:

      - Count3_en = 0

      - Count3_term = 0

      - Count2_en = 1

    - Count2 term high sets up next state to “NS_Left”

  - Then NS Left for 1 count

    - When state changes

      - Count1_en = 1

      - Count2_en = 0

      - Count2_term = 0

    - Count1 term high sets up next state to “NS_Green_short”

  - Then NS Green Short for 2 counts

    - When state changes

      - Count1_en = 0

      - Count1_term = 0

      - Count2_en = 1

    - Count1 term high sets up next state to “NS_Green_short”

  - Then NS Yellow

A

B

C

D

E

F

G

- Cycle through lights with EW Left turn bit on

  - Start at NS Yellow for 2 counts

  - Then EW Left for 1 count

  - Then EW Green Short for 2 counts

  - Then EW Yellow

A

D

C

B

- Cycle through lights with both directions left turn bit on

  - Start at EW Yellow for 3 counts

  - Then NS Left for 1 count

  - Then NS Green Short for 2 counts

  - Then NS Yellow for 2 counts

  - Then EW Left for 1 count

  - Then EW Green Short for 2 counts

  - Then back to EW Yellow

A

B

C

D

E

F

G

- Go into night mode

  - NS blinks Red 1 count on and 1 count off

  - EW blinks Yellow 1 count on and 1 count off

- Exit state mode and go back into normal operation

  - Begin at NS Green and continue normally

5A

5B

6A